## Work Method

| Task |  |
| --- | --- |
| Location/equipment/worksite |  |
| Work Party |  |
| Permit to Work ID No (if used) |  |
| Job Manager |  |
| Site Manager |  |
| Location |  |
| Description |  |
| Equipment ID ID/description/model |  |
| Date |  |

### Managing Hazards

| Hazard | Control Measure | Checked |
| --- | --- | --- |
| Unidentified Job Specific Hazards | Take 5 to be carried out by staff detailing on the day hazards. All staff to sign on to Take 5 accepting this form and the on the day hazards and controls. |  |
| - Lack of awareness of local asset & other - Simultaneous Operations | Project or Outage manager to notify work party of ALL Hydro Tasmania Assets in area (including Protection PLC, Mechanical, Electrical and Civil Assets). Work party to inspect and make safe. |  |

### General Requirements

| General Requirements |  |  |  |
| --- | --- | --- | --- |
| Hazard | Control Measure | N / A | Checked |
| Incorrect selection of equipment | Identify the type of work to be carried out and select the appropriate equipment for the task |  |  |
|  | Ensure that the torque (pressure setting) is known prior to selecting the appropriate equipment |  |  |
| Unserviceable condition of Hydraulic Torque Spanner | Ensure all of the hydraulic fittings are of the correct pressure rating |  |  |
|  | The hydraulic hoses are of the correct pressure rating and are in good condition |  |  |
|  | The condition of the Hydraulic Torque Spanner head is good (no signs of cracks or fatigue) |  |  |
|  | The pump and pressure gauge are in good condition |  |  |
|  | The reaction legs are in good condition (no signs of cracks or fatigue) and are the correct legs for the job |  |  |
|  | The pendant cable and pendant are in good condition |  |  |
|  | The pump motor electrical lead is in good condition and has been tested and tagged and is within the date range |  |  |
|  | Check pump oil level is correct and only use recommended hydraulic spanner oil for topping up and that the oil is clear and amber in colour. |  |  |
| Socket / spanner selection | Ensure that only approved and identified sockets (marked with orange paint band) / spanners are used |  |  |
|  | Inspect for damage, cracks etc before use |  |  |
|  | Ensure that the approved and identified socket / spanner selected is the correct fit to the nut or bolt before applying pressure |  |  |
| Torque setting selection | Ensure that the correct torque conversion chart accompanies the hydraulic torque spanner prior to the spanner being used |  |  |
|  | Ensure that the torque required for the job is known from drawings, specifications, engineering etc |  |  |
|  | Convert the torque setting to a pressure setting using the correct torque conversion chart for the particular type of hydraulic torque spanner being used |  |  |
| Testing the Hydraulic Torque Spanner equipment with socket or spanner attached prior to use | Connect the hydraulic torque spanner unit to the pump and check the security of the fittings |  |  |
|  | Set up the correct pressure in accordance with the hydraulic torque spanner operating manual instructions |  |  |
|  | Ensure the area around the hydraulic torque spanner is clear while the hydraulic torque spanner is being tested and all personnel in the area understands what is happening |  |  |
|  | Operate the hydraulic torque spanner on the floor to check correct mechanical operation, to set the correct pressure and check for oil leaks |  |  |
| Operation of Hydraulic Torque Spanner (communication) | The hydraulic torque spanner is to be operated by a minimum of two personnel who are authorised to use the hydraulic torque spanner |  |  |
|  | Where-ever practically possible the hydraulic torque spanner pump operator will be in line of sight with the hydraulic torque spanner head handler |  |  |
|  | Ensure all personnel not required during the hydraulic torque spanner operation are clear of area |  |  |
|  | The hydraulic torque spanner pump operator will not operate the hydraulic torque spanner equipment until given the “All Clear” command from the hydraulic torque spanner head handler |  |  |
|  | The hydraulic torque spanner pump operator will not operate the hydraulic torque spanner pump unit if they are not clear on the instructions that have been given by the hydraulic torque spanner head handler |  |  |
|  | The hydraulic torque spanner pump remote control operator will cease operation of the pump when given instruction by anyone who gives the command to stop |  |  |
| Use of Hydraulic Torque Spanner on site, varied locations and manual handling | Ensure there is a thorough understanding (detailed planning stage) of the issues involved with using the hydraulic torque spanner |  |  |
|  | Ensure a Take 5 is developed on-site in consultation with personnel carrying out the work clearly identifying the manual handling issues associated with using the hydraulic torque spanner equipment |  |  |
|  | Ensure a manual handling risk assessment is carried out for the work involved |  |  |
| Incorrect PPE | The minimum requirement for PPE will be safety helmet with high impact full face visor, gloves and steel capped safety shoes or boots |  |  |

Site Registration

| Certification |  |  |
| --- | --- | --- |
| I have personally checked and verified all items on all pages of this check list |  |  |
| Works Manager Name | Signature | Date |
| I acknowledge and agree that all items are detailed on all pages of this check list. I agree to ensure that all employees and sub-contractors engaged in this work will fully comply with all control measures indicated |  |  |
| Authorised Hydraulic Torque Spanner User | Signature | Date |

| Daily Review |  |  |
| --- | --- | --- |
| If There is a significant change in Work and or Conditions a new Check List is Required |  |  |
| Date and Time | Reviewed / Comment | New Check List Required |